A project report on

AUTONOMOUS PATTERN RECOGNIZING PAINTING SYSTEM

Submitted in partial fulfillment for the award of the degree of

Bachelor of Technology in Computer Science and Engineering

by

DAKSH AGARWAL (25BCE5098)

VARSHA S (25BCE5099)

ABHINAV SUNEESH (25BCE5100)

KARTHIK BILUKUMAR (25BLC1193)

VISHWAJITH VD (25BLC1191)

Under the guidance of

Dr. SARAVANAN P

Associate Professor

[INSERT VIT LOGO HERE]

SCHOOL OF COMPUTER SCIENCE AND ENGINEERING

VELLORE INSTITUTE OF TECHNOLOGY

Chennai Campus

January 2025

AUTONOMOUS PATTERN RECOGNIZING PAINTING SYSTEM

A Project Report

Submitted in partial fulfillment of the requirements

for the degree of

Bachelor of Technology

in

Computer Science and Engineering

by

DAKSH AGARWAL (25BCE5098)

VARSHA S (25BCE5099)

ABHINAV SUNEESH (25BCE5100)

KARTHIK BILUKUMAR (25BLC1193)

VISHWAJITH VD (25BLC1191)

SCHOOL OF COMPUTER SCIENCE AND ENGINEERING

VELLORE INSTITUTE OF TECHNOLOGY

Chennai Campus

January 2025

DECLARATION

I hereby declare that the project work entitled "AUTONOMOUS PATTERN RECOGNIZING PAINTING SYSTEM" submitted to the Vellore Institute of Technology, Chennai Campus, is a record of an original work done by me under the guidance of Dr. SARAVANAN P, Associate Professor, School of Computer Science and Engineering, VIT Chennai.

This project work has not been submitted to any other university or institution for the award of any degree, diploma, or certificate.

I also declare that this project work is my own effort and the results embodied in this report have not been submitted for any other award.

Place: Chennai						Signature of the Student

Date: ___________						(DAKSH AGARWAL)

CERTIFICATE

This is to certify that the project report entitled "AUTONOMOUS PATTERN RECOGNIZING PAINTING SYSTEM" submitted by DAKSH AGARWAL (25BCE5098), VARSHA S (25BCE5099), ABHINAV SUNEESH (25BCE5100), KARTHIK BILUKUMAR (25BLC1193), and VISHWAJITH VD (25BLC1191), to the Vellore Institute of Technology, Chennai Campus, in partial fulfillment of the requirements for the award of the degree of Bachelor of Technology in Computer Science and Engineering, is a bonafide record of the project work carried out by them under my guidance and supervision.

The contents of this report, in full or in parts, have not been submitted to any other university or institution for the award of any degree or diploma.

Date: ___________

Dr. SARAVANAN P

Associate Professor

School of Computer Science and Engineering

VIT Chennai

ABSTRACT

The painting industry has traditionally depended on manual labor, which presents challenges including physical strain, inconsistent coverage, health hazards from paint fumes, and inefficient resource utilization. This project presents the Autonomous Pattern Recognizing Painting System, an innovative solution that leverages computer vision and IoT technologies to automate wall painting operations.

The system employs an ESP32-CAM module as its core sensing unit, providing high-resolution image capture and WiFi connectivity in a compact, cost-effective package. A sophisticated multi-method weighted voting algorithm combines six detection techniques—adaptive threshold, relative brightness, low saturation detection, Otsu threshold, LAB lightness, and blurred threshold—to reliably identify unpainted areas across varying lighting conditions. The weighted combination achieves 93% detection accuracy, significantly outperforming individual methods.

The paint detection results inform an intelligent spray control system operating in three modes: precision mode for isolated unpainted spots, continuous mode for extended regions, and smart mode that automatically optimizes spray strategy based on spatial analysis of unpainted cells. A Flask-based backend server coordinates all operations, providing REST APIs for detection, spray control, and video streaming. The responsive web interface enables real-time monitoring, sensitivity adjustment, and manual intervention when necessary.

Testing demonstrated that the system reduces painting time by 50-60% compared to manual methods while minimizing paint wastage by 50-60% through targeted application. The 30-second watchdog timer and manual override controls ensure safe operation. The modular architecture facilitates future enhancements including drone integration for autonomous positioning, machine learning-based detection, and multi-color support.

Keywords: Computer Vision, Paint Detection, ESP32-CAM, Flask, IoT, Automation, OpenCV, Spray Control

ACKNOWLEDGEMENT

I would like to express my sincere gratitude to all those who contributed to the successful completion of this project.

First and foremost, I extend my heartfelt thanks to my project guide, Dr. SARAVANAN P, Associate Professor, School of Computer Science and Engineering, VIT Chennai, for his invaluable guidance, continuous support, and constructive suggestions throughout this project. His expertise and encouragement have been instrumental in shaping this work.

I am deeply grateful to the School of Computer Science and Engineering, VIT Chennai, for providing the necessary infrastructure and resources that enabled the development and testing of this project.

I would also like to thank my teammates VARSHA S, ABHINAV SUNEESH, KARTHIK BILUKUMAR, and VISHWAJITH VD for their collaborative efforts, technical discussions, and unwavering support throughout this journey.

I extend my appreciation to the faculty members and lab assistants who provided technical assistance and access to laboratory facilities.

Finally, I am thankful to my family and friends for their constant encouragement, understanding, and moral support that motivated me to complete this project successfully.

This project would not have been possible without the collective support of all these individuals, and I am truly grateful for their contributions.

DAKSH AGARWAL

TABLE OF CONTENTS

Declaration								i

Certificate								ii

Abstract								iii

Acknowledgement								iv

Table of Contents								v

List of Figures								vi

List of Tables								vii

List of Abbreviations								viii

CHAPTER 1: INTRODUCTION								1

1.1 Overview								1

1.2 Problem Statement								2

1.3 Objectives								3

1.4 Scope of the Project								4

1.5 Mapping to Sustainable Development Goals								5

CHAPTER 2: LITERATURE REVIEW								7

2.1 Existing Painting Systems								7

2.2 Computer Vision in Automation								8

2.3 ESP32-CAM Applications								9

2.4 Flask Web Applications for IoT								10

CHAPTER 3: METHODOLOGY								12

3.1 System Architecture								12

3.2 Hardware Design								13

3.3 Paint Detection Algorithm								14

3.4 Grid-Based Detection								16

3.5 Spray Control Modes								17

CHAPTER 4: IMPLEMENTATION								19

4.1 ESP32-CAM Firmware								19

4.2 Flask Backend Server								20

4.3 Paint Detection Implementation								22

4.4 Web Interface								24

4.5 Smart Spray Implementation								26

CHAPTER 5: RESULTS AND DISCUSSION								28

5.1 Testing Environment								28

5.2 Paint Detection Accuracy								29

5.3 Spray Control Performance								30

5.4 System Integration Testing								31

5.5 Comparison with Manual Painting								32

5.6 Discussion								33

CHAPTER 6: CONCLUSION AND FUTURE WORK								35

6.1 Conclusion								35

6.2 Future Work								36

REFERENCES								38

APPENDIX A: Code Listings								40

LIST OF FIGURES

Figure 1.1: Traditional vs Automated Painting Comparison					2

Figure 3.1: System Architecture Block Diagram					12

Figure 3.2: ESP32-CAM Pinout Diagram					13

Figure 3.3: Paint Detection Algorithm Flowchart					16

Figure 3.4: Detection Grid Visualization Example					17

Figure 4.1: ESP32-CAM Firmware Architecture					20

Figure 4.2: Flask Server Architecture					22

Figure 4.3: Web Interface Screenshot					25

Figure 4.4: Smart Spray Mode Selection Flowchart					27

Figure 5.1: Accuracy vs Sensitivity Graph					29

Figure 5.2: Painting Time vs Coverage Graph					31

Figure 5.3: System Performance Summary Chart					34

LIST OF TABLES

Table 3.1: Spray Mode Comparison						18

Table 5.1: Detection Method Accuracy Comparison						29

Table 5.2: Spray Mode Performance Comparison						30

LIST OF ABBREVIATIONS

API		Application Programming Interface

BFS		Breadth-First Search

CAM		Camera

CNN		Convolutional Neural Network

CORS		Cross-Origin Resource Sharing

ESP		Espressif

GPIO		General Purpose Input/Output

HSV		Hue, Saturation, Value

HTTP		Hypertext Transfer Protocol

IoT		Internet of Things

IP		Internet Protocol

JPEG		Joint Photographic Experts Group

LAB		Lightness, A, B (color space)

MJPEG		Motion JPEG

REST		Representational State Transfer

RGB		Red, Green, Blue

SDG		Sustainable Development Goal

SSE		Server-Sent Events

UI		User Interface

VIT		Vellore Institute of Technology

VOC		Volatile Organic Compound

WiFi		Wireless Fidelity

CHAPTER 1

INTRODUCTION

1.1 OVERVIEW

The painting industry has traditionally relied on manual labor for surface coating applications. Wall painting, in particular, presents significant challenges including physical strain on workers, inconsistent coverage, exposure to potentially harmful paint fumes, and the time-intensive nature of the process. In commercial and industrial settings, these challenges are amplified by the scale of operations and the need for consistent quality.

Automation in painting has been explored through various approaches, from simple spray machines to sophisticated robotic arms. However, most existing solutions focus on industrial applications and fail to address the unique requirements of wall painting, which demands adaptive coverage based on surface condition assessment.

The Autonomous Pattern Recognizing Painting System represents a novel approach to wall painting automation. Unlike conventional systems that operate blindly, this system employs computer vision to analyze the painting surface in real-time, identifying areas that require paint application. This intelligent approach minimizes paint wastage, ensures uniform coverage, and reduces the need for human intervention in potentially hazardous environments.

The system utilizes an ESP32-CAM module as its core sensing unit, combining a high-resolution camera with WiFi connectivity in a compact, cost-effective package. The camera captures images of the wall surface, which are then processed using a sophisticated multi-method paint detection algorithm. This algorithm employs six different techniques to identify unpainted (white) areas, weighted and combined to produce robust results across varying lighting conditions and surface textures.

[INSERT FIGURE 1.1: Traditional vs Automated Painting Comparison HERE]

The paint detection results inform the spray control system, which operates in multiple modes: Precision Mode targets individual cells for isolated unpainted areas, Continuous Mode maintains spray activation for adjacent unpainted regions, and Smart Mode automatically selects between precision and continuous modes based on the spatial distribution of unpainted cells.

A Flask-based web server acts as the central coordination hub, receiving camera data, executing paint detection algorithms, and controlling the spray mechanism. The responsive web interface enables operators to monitor system status, adjust detection sensitivity, and manually intervene when necessary.

1.2 PROBLEM STATEMENT

The traditional wall painting process faces several critical challenges that impact efficiency, quality, and worker safety:

1. Labor-Intensive Nature: Manual painting requires significant physical effort, particularly for large surfaces or elevated areas. Workers must maintain awkward positions for extended periods, leading to fatigue and musculoskeletal disorders. The repetitive nature of the task further compounds these physical challenges.

2. Inconsistent Coverage: Human painters, despite their best efforts, cannot achieve perfectly uniform paint application. Factors such as fatigue, varying pressure on brushes or rollers, and subjective assessment of coverage lead to inconsistencies. These variations result in visible imperfections, requiring additional coats and rework.

3. Health Hazards: Paint application exposes workers to volatile organic compounds (VOCs), fine particulates, and potentially toxic substances. Prolonged exposure can lead to respiratory issues, skin irritation, and long-term health complications. While personal protective equipment mitigates some risks, it adds to worker discomfort and may not be consistently used.

4. Time and Cost Inefficiency: Manual painting is inherently slow, particularly when high-quality finishes are required. Multiple coats often necessitate waiting periods for drying, extending project timelines. Labor costs constitute a significant portion of painting project expenses, particularly in regions with high wage rates.

5. Paint Wastage: Without precise control over application, manual methods result in excessive paint usage. Overspray, drips, and the need for touch-ups contribute to material wastage, increasing project costs and environmental impact.

6. Difficulty in Quality Assessment: Real-time assessment of paint coverage is challenging for human workers, particularly across large surfaces. Missed spots or insufficient coverage may only become apparent after completion, requiring costly rework.

1.3 OBJECTIVES

The primary objectives of this project are:

1. To develop an intelligent paint detection system capable of identifying unpainted areas on wall surfaces using computer vision techniques, achieving detection accuracy exceeding 90% across varying lighting conditions.

2. To implement a multi-method weighted voting algorithm that combines six different detection techniques (adaptive threshold, relative brightness, saturation analysis, Otsu threshold, LAB lightness, and blur-based detection) for robust unpainted area identification.

3. To design and develop ESP32-CAM firmware that provides real-time camera capture, WiFi connectivity, and spray relay control with safety features including a 30-second watchdog timer for continuous spray mode.

4. To create a Flask-based backend server providing REST APIs for paint detection, spray control, and video stream proxying, enabling seamless integration between hardware and user interface components.

5. To build a responsive web interface offering real-time camera feed display, detection grid visualization, spray control buttons, sensitivity adjustment, and mission logging capabilities.

6. To implement multiple spray modes including precision mode for isolated areas, continuous mode for extended regions, and smart mode that automatically optimizes spray strategy based on unpainted cell distribution.

7. To achieve efficient paint utilization by targeting only unpainted areas, reducing paint wastage compared to traditional blanket application methods.

8. To ensure system safety through multiple safeguards including relay watchdog timers, manual override controls, and emergency stop functionality.

1.4 SCOPE OF THE PROJECT

The scope of the Autonomous Pattern Recognizing Painting System encompasses the following domains:

Technical Scope

Hardware Components: ESP32-CAM (AI Thinker) module for camera capture and WiFi connectivity, Relay module (GPIO 13) for spray mechanism control, Spray mechanism (solenoid valve or motor-driven pump), and 5V power supply for ESP32-CAM operation.

Software Components: ESP32-CAM firmware (Arduino/C++), Flask backend server (Python), Web interface (HTML, CSS, JavaScript), and OpenCV for image processing.

Communication: WiFi Access Point mode (SSID: PaintSystem), HTTP/REST API communication, and MJPEG video streaming.

Application Scope

The system is designed for indoor wall painting applications, white/light-colored unpainted surface detection, single color paint application, and medium-scale wall surfaces (up to 3m × 3m in current configuration).

Limitations (Out of Scope)

The following are outside the current project scope: outdoor applications with variable lighting, multi-color paint application, textured or patterned surface analysis, autonomous navigation or positioning (manual placement required), and large-scale commercial/industrial applications.

1.5 MAPPING TO SUSTAINABLE DEVELOPMENT GOALS

This project aligns with multiple United Nations Sustainable Development Goals (SDGs):

SDG 3 - Good Health and Well-Being

The Autonomous Pattern Recognizing Painting System reduces worker exposure to paint fumes and hazardous chemicals by automating the painting process. By minimizing the need for workers to be in close proximity to freshly applied paint, the system contributes to improved occupational health outcomes. The precision application also reduces overspray, limiting environmental contamination.

SDG 9 - Industry, Innovation and Infrastructure

The project demonstrates innovative application of computer vision and IoT technologies to traditional industries. The integration of ESP32-CAM, Flask backend, and intelligent algorithms represents Industry 4.0 principles applied to construction and maintenance sectors. This technological advancement supports industrial innovation and infrastructure improvement.

SDG 11 - Sustainable Cities and Communities

Efficient painting systems contribute to better-maintained urban infrastructure. By improving the quality and durability of painted surfaces, the system supports sustainable building maintenance. The reduced time and cost for painting operations enable more frequent maintenance cycles, contributing to improved urban aesthetics.

SDG 12 - Responsible Consumption and Production

The intelligent paint detection and targeted application significantly reduce paint wastage. By identifying only areas requiring paint and applying precise amounts, the system supports responsible material consumption. Reduced wastage also means fewer paint containers requiring disposal, minimizing environmental impact.

CHAPTER 2

LITERATURE REVIEW

2.1 EXISTING PAINTING SYSTEMS

The evolution of painting systems has progressed from purely manual methods to various levels of automation. This section reviews existing approaches and their limitations.

Manual Painting Methods

Traditional painting relies on brushes, rollers, and manual spray guns operated by skilled workers. While these methods offer flexibility and are suitable for intricate work, they suffer from physical strain on workers, particularly for overhead and elevated surfaces; inconsistent quality dependent on individual skill and fatigue levels; time-intensive processes with limited throughput; and health risks from prolonged exposure to paint chemicals.

Airless Spray Systems

High-pressure airless spray systems increase painting speed by atomizing paint through small orifices. These systems improve efficiency but still require manual operation and skill. They provide faster coverage compared to brushes and rollers, better penetration into surface irregularities, but result in higher paint consumption due to overspray and require operator training for optimal results.

Robotic Painting Arms

Industrial robotic arms have been employed for painting in manufacturing settings, particularly in automotive industries. They offer precise, repeatable movements and consistent coverage quality. However, they require high initial investment costs, have limited flexibility for variable surfaces, and are primarily suited for structured factory environments.

Paint Robots for Construction

Recent developments include robots designed for construction painting applications such as wall-climbing robots with spray attachments, rail-mounted painting systems, and collaborative robots (cobots) for painting tasks. However, these systems typically lack intelligent surface assessment capabilities, applying paint uniformly regardless of existing coverage.

2.2 COMPUTER VISION IN AUTOMATION

Computer vision has transformed numerous automation applications by enabling machines to "see" and interpret their environment. This section reviews relevant developments.

Image Processing Fundamentals

Basic image processing techniques form the foundation of computer vision systems. Color space transformations (RGB, HSV, LAB) enable analysis of different color properties. Thresholding provides binary segmentation based on intensity or color values. Edge detection identifies boundaries and discontinuities. Morphological operations clean and refine detection results.

Adaptive Thresholding

Unlike global thresholding, adaptive methods calculate threshold values locally. This accounts for varying illumination across images. Gaussian and mean-based adaptive methods are particularly effective for uneven lighting conditions. Research by Sezgin and Sankur (2004) demonstrated that adaptive thresholding outperforms global methods for images with non-uniform illumination.

Multi-Method Detection Approaches

Combining multiple detection techniques improves robustness. Ensemble methods combine predictions from multiple algorithms. Weighted voting assigns importance to different techniques. Consensus-based detection requires agreement among methods. Studies have shown that multi-method approaches achieve higher accuracy than single-technique methods, particularly in challenging conditions.

2.3 ESP32-CAM APPLICATIONS

The ESP32-CAM module has emerged as a popular choice for IoT and embedded vision applications due to its integration of camera, WiFi, and processing capabilities.

ESP32-CAM Specifications

The AI Thinker ESP32-CAM module features an ESP32-S chip with dual-core processor, OV2640 2-megapixel camera, built-in WiFi and Bluetooth, SD card slot for storage, multiple GPIO pins for peripheral control, and low power consumption.

Camera-Based IoT Projects

ESP32-CAM has been utilized in various applications including security cameras with motion detection and remote monitoring, wildlife cameras with triggered capture and low power modes, QR/barcode scanning for automated data capture systems, face recognition for access control systems, and machine vision for quality inspection and counting systems.

2.4 FLASK WEB APPLICATIONS FOR IoT

Flask, a lightweight Python web framework, has become popular for IoT backend development due to its simplicity and flexibility.

Flask Framework Overview

Flask provides essential features for web application development including routing for URL handling, template rendering for dynamic pages, request/response handling, and an extension ecosystem for added functionality.

Flask for IoT Applications

Flask serves as an effective backend for IoT systems, providing device management for tracking connected devices, data collection for receiving sensor data via APIs, control interfaces for sending commands to actuators, and real-time monitoring through WebSocket and SSE support. Research indicates that Flask-based backends handle moderate IoT workloads efficiently while maintaining code simplicity.

CHAPTER 3

METHODOLOGY

3.1 SYSTEM ARCHITECTURE

The Autonomous Pattern Recognizing Painting System employs a three-tier architecture comprising hardware, backend processing, and user interface layers.

[INSERT FIGURE 3.1: System Architecture Block Diagram HERE]

Hardware Layer

The hardware layer consists of the ESP32-CAM module and spray control mechanism. The ESP32-CAM Module acts as WiFi Access Point (SSID: PaintSystem, Password: paintdrone123) with IP Address 192.168.4.1. Port 80 handles HTTP control and capture, while Port 81 provides MJPEG video stream. The Spray Control uses a relay module on GPIO 13 with active-low configuration (LOW = spray on) and includes a 30-second watchdog timer for safety.

Backend Layer

The Flask server processes images and coordinates system operation. Image Capture retrieves frames from ESP32-CAM. Paint Detection analyzes images using the multi-method algorithm. Grid Processing divides the image into 96 cells (8×12). Spray Coordination controls spray timing and patterns. API Provision offers REST endpoints for UI communication.

User Interface Layer

The responsive web interface provides real-time camera stream display, detection grid visualization, spray control buttons, sensitivity adjustment slider, and mission progress logging.

3.2 HARDWARE DESIGN

ESP32-CAM Module

The AI Thinker ESP32-CAM was selected for its integrated camera and WiFi capabilities.

| Specification | Value |
| --- | --- |
| Processor | ESP32-S (Dual-core, 240MHz) |
| Camera | OV2640, 2MP |
| WiFi | 802.11 b/g/n |
| RAM | 520KB SRAM + 4MB PSRAM |
| Flash | 4MB |
| GPIO | Multiple pins including GPIO 13 for relay |

[INSERT FIGURE 3.2: ESP32-CAM Pinout Diagram HERE]

Relay Module Connection

The spray mechanism is controlled via a relay connected to GPIO 13. GPIO 13 connects to Relay IN, 5V to Relay VCC, and GND to Relay GND. The relay switches the spray solenoid valve or pump motor when activated.

3.3 PAINT DETECTION ALGORITHM

The paint detection system employs a sophisticated multi-method weighted voting algorithm to identify unpainted (white) areas reliably across varying conditions.

Algorithm Overview

Six detection methods analyze the image independently, each contributing a weighted vote:

| Method | Weight | Purpose |
| --- | --- | --- |
| Adaptive Threshold | 30% | Local intensity analysis |
| Relative Brightness | 20% | Value channel brightness |
| Low Saturation | 25% | Color desaturation detection |
| Otsu Threshold | 10% | Automatic global threshold |
| LAB Lightness | 10% | Perceptual brightness |
| Blurred Threshold | 5% | Noise-resistant detection |

Method 1 - Adaptive Threshold (30%): Analyzes local intensity variations using Gaussian adaptive thresholding. This method excels at handling non-uniform illumination by calculating thresholds locally.

Method 2 - Relative Brightness (20%): Converts to HSV color space and analyzes the Value (brightness) channel. Effective for detecting high-brightness white areas.

Method 3 - Low Saturation Detection (25%): White areas exhibit low color saturation. This method identifies desaturated regions and, combined with brightness, provides robust white detection.

Method 4 - Otsu Threshold (10%): Automatic threshold calculation using Otsu's method provides a statistically optimal global threshold.

Method 5 - LAB Lightness (10%): The LAB color space separates lightness from color. LAB lightness correlates better with human perception of brightness.

Method 6 - Blurred Threshold (5%): Applies Gaussian blur before thresholding to reduce noise sensitivity, reducing false positives from image noise and artifacts.

Weighted Combination

The six methods are combined using weighted voting. A pixel is classified as unpainted if the weighted sum exceeds 40%.

[INSERT FIGURE 3.3: Paint Detection Algorithm Flowchart HERE]

3.4 GRID-BASED DETECTION

Grid Configuration

The image is divided into a grid for cell-by-cell analysis with 8 Grid Rows, 12 Grid Columns, and 96 Total Cells.

Cell Classification

Each cell is analyzed independently: extract cell region from image, apply multi-method detection to cell pixels, calculate percentage of unpainted pixels, and mark cell unpainted if percentage ≥ 40%.

Grid Visualization

The detection results are visualized as colored overlays. Unpainted cells are shown in Red (RGB: 255, 0, 0) and Painted cells in Green (RGB: 0, 255, 0).

[INSERT FIGURE 3.4: Detection Grid Visualization Example HERE]

3.5 SPRAY CONTROL MODES

Mode 1 - Precision Spray: Single-cell targeting for isolated unpainted areas with 1 second spray duration per cell and 0.5 seconds delay between cells. Best for scattered unpainted spots.

Mode 2 - Continuous Spray: Extended spray for adjacent unpainted regions. Maintains spray while traversing unpainted cells with 0.3 seconds delay between cells. Best for large unpainted areas.

Mode 3 - Smart Spray: Intelligent mode selection based on spatial analysis. Counts unpainted cells and detects clusters. If clusters detected, uses continuous mode; if isolated cells, uses precision mode. Employs hybrid approach for mixed patterns. The smart mode analyzes unpainted cell distribution using 4-connectivity (adjacent cells share edges).

CHAPTER 4

IMPLEMENTATION

4.1 ESP32-CAM FIRMWARE

The ESP32-CAM firmware provides camera capture, WiFi connectivity, and relay control functionality.

WiFi Access Point Configuration

The ESP32-CAM operates in Access Point mode with SSID "PaintSystem" and password "paintdrone123". The IP address is set to 192.168.4.1. This configuration allows direct connection without requiring an existing WiFi network.

Dual HTTP Server

Two HTTP servers handle different functions. Port 80 serves as the Control Server providing /capture for JPEG images, /spray/on to activate relay, /spray/off to deactivate relay, and a root page with status information. Port 81 serves as the Stream Server providing MJPEG video stream for live preview with Content-Type: multipart/x-mixed-replace and boundary-delimited JPEG frames.

Relay Control with Watchdog

GPIO 13 controls the spray relay with safety features. The relay uses active-low configuration and includes a 30-second watchdog timeout. The watchdog timer automatically stops spraying after 30 seconds if no new commands are received.

[INSERT FIGURE 4.1: ESP32-CAM Firmware Architecture HERE]

4.2 FLASK BACKEND SERVER

The Python Flask server handles image processing, paint detection, and spray coordination.

Core Endpoints

| Endpoint | Method | Function |
| --- | --- | --- |
| / | GET | Serve web interface |
| /video_feed | GET | Proxy MJPEG stream |
| /capture | POST | Capture and analyze image |
| /spray | POST | Single spray pulse |
| /spray_start | POST | Start continuous spray |
| /spray_stop | POST | Stop spray |
| /spray_sequence | GET | SSE precision spray |
| /smart_spray_sequence | GET | SSE smart spray |
| /ping | GET | Check ESP32-CAM status |

Video Feed Proxy

The server proxies the ESP32-CAM stream for CORS compatibility. It retrieves the stream from the ESP32-CAM at port 81 and forwards it to the web client with appropriate content type headers.

Server-Sent Events for Spray Sequences

Spray sequences use SSE for real-time progress updates. For each unpainted cell, the system activates the spray, waits for the configured duration, deactivates the spray, and sends a progress update to the client.

[INSERT FIGURE 4.2: Flask Server Architecture HERE]

4.3 PAINT DETECTION IMPLEMENTATION

PaintDetector Class

The detection logic is encapsulated in a PaintDetector class with grid configuration of 8 rows and 12 columns, and an unpainted threshold of 0.40 (40%). The detect method performs multi-method analysis, grid division, cell classification, and overlay generation.

Grid Division and Cell Analysis

The image is divided into cells based on grid dimensions. For each cell, the system extracts the cell region, applies the combined detection mask, calculates the white ratio (percentage of unpainted pixels), and marks the cell as unpainted if the ratio exceeds 40%.

4.4 WEB INTERFACE

HTML Structure

The responsive web interface uses Bootstrap for styling. The layout includes a Camera Feed Panel (col-md-8) displaying the camera feed image and detection overlay canvas, and a Control Panel (col-md-4) with Capture & Detect button, sensitivity slider, spray control buttons (Start/Stop/Smart), and a Mission Log section for progress display.

JavaScript Functionality

Client-side JavaScript handles API communication. The captureAndDetect function sends POST requests to /capture with sensitivity settings and displays the returned grid and overlay. The startSmartSpray function creates an EventSource for /smart_spray_sequence and handles progress updates via SSE.

[INSERT FIGURE 4.3: Web Interface Screenshot HERE]

4.5 SMART SPRAY IMPLEMENTATION

Cluster Detection Algorithm

The smart spray mode identifies clusters of unpainted cells using breadth-first search (BFS). The algorithm extracts unpainted cell coordinates, initializes a visited set and clusters list, and for each unvisited unpainted cell performs BFS to find all connected cells. Adjacent cells are determined using 4-connectivity (up, down, left, right neighbors).

Mode Selection Logic

Based on cluster analysis, the appropriate spray mode is selected. For clusters with 3 or more cells, continuous mode is used for efficiency. For isolated cells (fewer than 3), precision mode is used for accuracy.

[INSERT FIGURE 4.4: Smart Spray Mode Selection Flowchart HERE]

CHAPTER 5

RESULTS AND DISCUSSION

5.1 TESTING ENVIRONMENT

Hardware Setup

| Component | Specification |
| --- | --- |
| ESP32-CAM | AI Thinker module, OV2640 camera |
| Relay | 5V single-channel relay module |
| Spray Mechanism | 12V solenoid valve |
| Power Supply | 5V 2A USB adapter |
| Test Surface | 2m × 1.5m whiteboard with paint patches |

Software Environment

| Component | Version |
| --- | --- |
| ESP32 Arduino Core | 2.0.14 |
| Python | 3.10.0 |
| Flask | 2.3.0 |
| OpenCV | 4.8.0 |
| NumPy | 1.24.0 |

Test Conditions

Testing was conducted under various lighting conditions: Bright artificial light (500 lux), Moderate indoor light (200 lux), and Low light conditions (50 lux).

5.2 PAINT DETECTION ACCURACY

Method Comparison

Individual detection methods were evaluated for accuracy:

| Method | Bright Light | Moderate Light | Low Light | Average |
| --- | --- | --- | --- | --- |
| Adaptive Threshold | 94% | 91% | 82% | 89% |
| Relative Brightness | 92% | 88% | 75% | 85% |
| Low Saturation | 90% | 89% | 87% | 89% |
| Otsu Threshold | 88% | 83% | 70% | 80% |
| LAB Lightness | 91% | 89% | 84% | 88% |
| Blurred Threshold | 85% | 82% | 78% | 82% |
| Combined (Weighted) | 96% | 94% | 88% | 93% |

The weighted combination consistently outperformed individual methods, demonstrating the effectiveness of the multi-method approach.

[INSERT FIGURE 5.1: Accuracy vs Sensitivity Graph HERE]

5.3 SPRAY CONTROL PERFORMANCE

Response Time

| Operation | Average Time | Std Dev |
| --- | --- | --- |
| Relay ON command | 45ms | 8ms |
| Relay OFF command | 42ms | 7ms |
| Capture to spray | 320ms | 25ms |
| Full cell spray cycle | 1.5s | 0.1s |

The system demonstrated consistent, low-latency control suitable for real-time operation.

Spray Mode Comparison

| Metric | Precision Mode | Continuous Mode | Smart Mode |
| --- | --- | --- | --- |
| Time per cell | 1.5s | 0.8s | Variable |
| Paint accuracy | 95% | 92% | 94% |
| Paint efficiency | 98% | 85% | 93% |
| Best use case | Isolated spots | Large areas | Mixed |

5.4 SYSTEM INTEGRATION TESTING

End-to-End Performance

| Test Surface | Unpainted Cells | Total Time | Cells/Minute |
| --- | --- | --- | --- |
| 25% unpainted | 24/96 | 42s | 34 |
| 50% unpainted | 48/96 | 78s | 37 |
| 75% unpainted | 72/96 | 108s | 40 |

[INSERT FIGURE 5.2: Painting Time vs Coverage Graph HERE]

5.5 COMPARISON WITH MANUAL PAINTING

| Metric | Manual Painting | Automated System |
| --- | --- | --- |
| Detection accuracy | 80-90% (subjective) | 93% (measured) |
| Time for 1m² | 5-8 minutes | 2-3 minutes |
| Paint wastage | 15-25% | 5-10% |
| Consistency | Variable | Uniform |

Advantages Observed

1. Consistent quality: Uniform detection threshold across entire surface. 2. Reduced fatigue: No physical strain on operator. 3. Objective assessment: Data-driven coverage decisions. 4. Documentation: Complete log of painted areas.

5.6 DISCUSSION

The weighted voting approach proved highly effective. Individual methods showed varying performance across lighting conditions. Combining methods compensated for individual weaknesses. The 40% threshold provided good balance between sensitivity and specificity.

The 8×12 grid division offered advantages: reduced computational complexity compared to pixel-level analysis, natural mapping to spray pattern planning, and intuitive visualization for operators.

The intelligent mode selection demonstrated practical benefits: reduced total painting time by 15-20% compared to uniform precision mode, maintained accuracy while improving efficiency, and automated decision-making reduced operator cognitive load.

[INSERT FIGURE 5.3: System Performance Summary Chart HERE]

CHAPTER 6

CONCLUSION AND FUTURE WORK

6.1 CONCLUSION

The Autonomous Pattern Recognizing Painting System successfully demonstrates the application of computer vision to automate paint detection and targeted spray control. The key achievements of this project include:

1. Robust Paint Detection: The multi-method weighted voting algorithm achieved 93% average accuracy across varying lighting conditions. By combining six different detection techniques, the system provides reliable unpainted area identification that surpasses any single method.

2. Effective Hardware Integration: The ESP32-CAM module proved to be an excellent platform, providing integrated camera, WiFi, and GPIO capabilities in a compact, cost-effective package. The dual-port HTTP server architecture enables simultaneous video streaming and control commands.

3. Intelligent Spray Control: The implementation of precision, continuous, and smart spray modes offers flexibility for different painting scenarios. The smart mode's cluster detection algorithm automatically optimizes spray strategy, reducing total painting time while maintaining coverage accuracy.

4. User-Friendly Interface: The responsive web interface provides real-time monitoring, intuitive controls, and comprehensive logging. Operators can adjust sensitivity, monitor progress, and intervene when necessary.

5. Safety Features: The 30-second watchdog timer and manual override controls ensure safe operation, preventing uncontrolled spray activation and enabling immediate response to unexpected situations.

6. Practical Performance: Testing demonstrated significant improvements over manual painting methods: 93% detection accuracy vs. 80-90% subjective human assessment, 50-60% reduction in painting time, and 50-60% reduction in paint wastage through targeted application.

6.2 FUTURE WORK

Several enhancements are planned for future development:

Hardware Improvements

1. Drone Integration: Future versions will incorporate autonomous drone capabilities for automated positioning, coverage of elevated areas, GPS-based waypoint navigation, and altitude-aware spray pressure adjustment.

2. Multi-Camera System: Implementing multiple cameras for stereo vision depth perception, 360-degree surface coverage, and improved edge detection.

3. Enhanced Spray Mechanism: Upgrading to variable-pressure spray control, multi-nozzle array for faster coverage, and paint color changing capability.

Software Enhancements

4. Machine Learning Integration: Replacing rule-based detection with Convolutional Neural Networks, transfer learning, and online learning for environment adaptation.

5. Multi-Color Detection: Extending capabilities to distinguish between different paint colors, support touch-up of specific colors, and detect paint degradation.

6. Path Planning Optimization: Implementing traveling salesman-based route optimization, energy-efficient spray sequences, and obstacle-aware navigation.

Application Extensions

7. Mobile Application: Developing companion apps for iOS and Android platforms with remote monitoring and push notifications.

8. Cloud Integration: Adding cloud capabilities for remote diagnostics, usage analytics, and multi-device coordination.

9. Industrial Applications: Scaling for large-scale commercial painting, automotive refinishing, and industrial coating applications.

The modular architecture of the current system facilitates these future enhancements, allowing incremental improvements without fundamental redesign.

REFERENCES

[1] Bradski, G., & Kaehler, A. (2008). Learning OpenCV: Computer Vision with the OpenCV Library. O'Reilly Media.

[2] Espressif Systems. (2023). ESP32 Technical Reference Manual. Espressif Systems.

[3] Grinberg, M. (2018). Flask Web Development: Developing Web Applications with Python (2nd ed.). O'Reilly Media.

[4] Otsu, N. (1979). A Threshold Selection Method from Gray-Level Histograms. IEEE Transactions on Systems, Man, and Cybernetics, 9(1), 62-66.

[5] Sezgin, M., & Sankur, B. (2004). Survey over image thresholding techniques and quantitative performance evaluation. Journal of Electronic Imaging, 13(1), 146-165.

[6] Sobel, I., & Feldman, G. (1968). A 3x3 isotropic gradient operator for image processing. Stanford Artificial Intelligence Project.

[7] Suzuki, S., & Abe, K. (1985). Topological structural analysis of digitized binary images by border following. Computer Vision, Graphics, and Image Processing, 30(1), 32-46.

[8] AI Thinker. (2020). ESP32-CAM Development Board User Manual. AI Thinker Technology.

[9] OpenCV Team. (2023). OpenCV Documentation. Retrieved from https://docs.opencv.org/

[10] Python Software Foundation. (2023). Python 3 Documentation. Retrieved from https://docs.python.org/3/

[11] Pallets Projects. (2023). Flask Documentation. Retrieved from https://flask.palletsprojects.com/

[12] Arduino. (2023). Arduino Reference. Retrieved from https://www.arduino.cc/reference/

APPENDIX A

CODE LISTINGS

A.1 ESP32-CAM Firmware (esp32cam.ino)

[Full firmware code available in repository: esp32cam/esp32cam.ino]

A.2 Flask Backend (app.py)

[Full backend code available in repository: backend/app.py]

A.3 Web Interface (index.html)

[Full interface code available in repository: backend/static/index.html]

[END OF DOCUMENT]